LUX13HS, LUX19HS, LUX160

A family of High Speed CMOS Image sensors with low noise Global Shutter pixel based on patented LUXIMATM CDS-in- pixel technology. The family includes 1Mpix, 2Mpix (full HD), and 16Mpix (4K) sensor developed for machine vision, 3D imaging, TV sports, and industrial markets.  The sensors share similar package allowing customers to design 3 cameras with essentially one design.

|  |  | LUX13HS |  | LUX19HS |  | LUX160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Optical format |  | 4/3” |  | 4/3” |  | 4/3” |  |
| Active resolution |  | 1280x864 |  | 1920x1080 |  | 4704x3424 |  |
| Pixel type |  | 7T shutter with CDS |  | 7T shutter with CDS |  | 5T shutter with CDS |  |
| Pixel pitch |  | 13.7um |  | 10um |  | 3.9um |  |
| Saturation/ Noise |  | 20,000e-/ 14e- |  | 15,000e-/ 10e- |  | 10,000e-/ 5-10e- |  |
| Responsivity |  | 25V/Lux-s @ 550nm |  | 20V/Lux-s @ 550nm |  | 4.5V/Lux-s @ 550nm |  |
| QE |  | >45% @ 550nm |  | >50% @ 550nm |  | >50% @ 550nm |  |
| Conversion gain |  | 70 uV/e- |  | 80 uV/e- |  | 120 uV/e- |  |
| DSNU |  | 4 mV r.m.s. |  | 4 mV r.m.s |  | 4 mV r.m.s. |  |
| PRNU |  | <1.5% rms |  | <1.5% rms |  | 2% rms |  |
| Shutter efficiency |  | 99.9% |  | 99.9% |  | 99.9% |  |
| Nominal Frame Rate @ Full resolution |  | 3800 Frames/s |  | 2500 Frames/s |  | 350 Frames/s |  |
| Column ADC |  | 10b |  | 10b |  | 10b |  |
| Data Output |  | 80 LVDS ports 10b 64 LVDS ports 8-b |  | 80 LVDS ports 10b 64 LVDS ports 8-b |  | 80 LVDS ports 10b 64 LVDS ports 8-b |  |
| Multiplexed Output |  | YES |  | YES |  | YES |  |
| Nominal clock rate |  | 133 MHz |  | 87.5 MHz |  | 90 MHz |  |
| Power supply |  | 3.3V/1.8V |  | 3.3V/1.8V |  | 3.3V/1.8V |  |
| Power consumption |  | 3W |  | 2.5W |  | 3W |  |
| Package |  | 345uPGA, 361uPGA |  | 345uPGA |  | 361uPGA, 345uPGA |  |
| Color Filter |  | RGB or Mono |  | RGB or Mono |  | RGB or Mono |  |